LEICESTER ROAD, LOUGHBOROUGH

Conservation Area Management Plan

Introduction

The local authority is required to formulate and publish proposals for preservation and enhancement of Conservation Areas. The first stage is to produce a “Conservation Area Character Appraisal”, which aims to give a clear assessment of the special interest, character and appearance that justified the area being designated.  It also establishes what is locally distinctive about Leicester Road and what needs to be protected in order to maintain its special character.  It is intended that this appraisal will provide a sound basis, defensible on appeal, for development control decisions and for the guidance of residents and developers.

This “Management Plan” is the next stage.  It gives design guidance that is applicable to Leicester Road and identifies opportunities for preservation and enhancement, where additional policy guidance or enforcement action maybe required and the need for a review of the conservation area boundary.

Background policy guidance

The following briefly sets out the planning policy background within which this guidance is written.

The Department for Communities & Local Government

PPS1 Delivering Sustainable Development

This guidance emphasises that high quality development through good design makes a vital contribution to the creation of safe, sustainable and livable communities.  Development proposals should not be conceived as short term and should be judged by their long-term impact.

By Design

Advocates the use of design guides to steer development proposals and the use of guidelines enables the Council to communicate its commitment to all in the development process.

PPG15 Planning and the Historic Environment

Sets out the need for Local Authorities to formulate and publish proposals for the preservation and enhancement of conservation areas.

Regional guidance

Planning Guidance for the East Midlands advocates the use of ‘design led’ approaches to development, which take account of historic character.  It also stresses the need for architectural design to be functional and to support vitality, yet respect local building character.

#### Structure Plan

Environment Policy 1; indicates the need to identify measures to enhance areas, sites and settings of architectural and historic interest.

#### Charnwood Community Strategy

The Community Strategy identifies that the quality of the built environment makes an important contribution to the economy of the Borough.

#### Charnwood Corporate Plan

This document identifies the aims and objectives of the Council to improve the built environment for everyone.  As a priority the Council seeks to protect the look and feel of our towns and villages.

Local Plan

The Local Plan sets out the basis on which the Council will view and determine planning applications.  Policy ST/1 ii and iii state the Councils aim to protect and improve the quality of the built environment of Charnwood. In the majority of cases the following policies may need to be considered before any design is undertaken.

- EV/1 Design

- EV/2 and 3 Archaeology

- EV/4 to 7 Listed buildings

- EV/8 Buildings of local architectural and historic interest

- EV/10 Development in conservation areas

- EV/11 to 13 Advertisments

- EV/18 Open spaces of special character

- EV/21 to 26 Ecological, geological and wildlife conservation

- EV/38 Satellite dishes

- CA/11 Use of upper floors

- CA/12 and 13 Shopfront design and security

Appendix 1 sets out these policies in full.

Leading in Design

This document builds on the design policies set out in the Charnwood Local Plan and will contribute to the development of more effective approaches to securing good design in the emerging Local Development Framework. The guide is also intended to support the implementation of the community strategy, Charnwood Together, by providing a set of principles that will inform the physical development implicit in the initiatives and actions of all partners in the local strategic partnership.

Other SPG/SPD guidance

- Draft Town Centre Masterplan

- Draft Hospital/Auberry Gap Development Brief.

- House Extensions

- Shopfronts & Signs

General Principles

Any proposed changes should be carried out in a sensitive manner, taking into account this established character.  New development must respond to its immediate environment i.e. its context, in terms of scale, form, materials and detailing.  Otherwise, alterations will have a detrimental effect on the historic and locally distinctive form of the village.

Within the conservation area, where the quality of the general environment is already considered to be high, the Council will insist on good quality schemes which respond positively to their historic setting, this extends to small buildings such as garages and even boundary walls and fences.  Even minor alterations need to be carefully considered as incremental change can have a significant detrimental affect on the character of an area over a period of time.

Central government guidance contained in PPS1 and PPG 15, Borough of Charnwood Local Plan, Leading in Design and other SPD will be used to assess the quality of proposal for new development.

The following general principles should be noted when considering any development in all parts of the conservation area:

- The conservation area has a distinct “grain” or pattern of built form and spaces which are part of its historic development.  This gives the conservation area great individuality, characterised by the pattern of historic buildings, ancient footpaths and highways and clearly defined boundaries.  This “grain” is an important part of the character of the conservation area and will be protected.

- The emphasis for new proposals will be on high quality of design.  There may be opportunity for innovative modern design. However a dramatic contemporary statement is unlikely to be appropriate.  Good modern design can be used to create positive changes in historic settlements

- Scale is the combination of a building’s height and bulk when related to its surroundings.  Proposed new development must take into account the scale of the existing buildings, and must not dominate of overwhelm them.

- Alterations and extensions must respect the form of the original building and its locality.  The use of high quality materials and detailing, whether modern or traditional is essential.  Roof lines, roof shape, eaves details, verge details and the creation of new chimneys are important considerations.

- Windows and doors of a traditional design respect the historic nature of the buildings to which they belong and make a very important contribution to the character and appearance of the conservation area.  The use of upvc and standardised high speed joinery techniques nearly always leads to unsuitably detailed windows which will be generally unacceptable in the conservation area. In most cases the building regulation requirements can be met without the need to use clumsy and awkwardly detailed windows.

- The appraisal has identified the types of materials that characterise the conservation area and where possible this should be used to help alterations respect that established character.

- Applicants for planning permission must provide a “Design Statement”, to explain the design decisions that have been made and to show how proposed alterations relate to their context.  A detailed analysis of the locality should demonstrate that there is a full appreciation of the local streetscape and how it has developed, including prevailing building forms, materials and plot ratios.

Procedures to ensure consistent decision-making

As mentioned previously the basis of the character appraisal is to inform and guide development control decisions.  A consistent approach to this decision making will be aided by providing:

- Conservation and design surgeries to help development control officers to make informed decisions, no matter how minor the proposed changes.

- Opportunities for pre-application discussion regarding significant alterations.

- Opportunities to review decisions and assess the impact of approved alterations through post development site visits.

Enforcement strategy

Effective enforcement is vital to make sure there is public confidence in the planning system to protect the special character of the area.  Unauthorised development can often be damaging to that character.

Taking proactive action can improve the appearance and character of the area, making it more attractive and in some instances increasing the potential for investment.  Effective monitoring of building work to make sure it is carried out in accordance with the approved details and with planning conditions ensures new development makes the positive contribution envisaged when permission was granted.

In order to protect the character of the conservation area the Borough Council will seek to:

- use enforcement powers in cases where unauthorised development unacceptably affects the character of the conservation area.

- take proactive action to improve or enhance the appearance of the area.

- monitor development under way to make sure it fully complies with the terms of any planning permission or listed building consent.

Carrying out unauthorised work to a listed building or to protected trees and hedgerows and the unauthorised demolition a building within a conservation area is an offence.  In such cases, the Council will consider prosecution of anyone responsible and any necessary remedial action.

Article 4 Direction proposals

The quality of the conservation area is often threatened by the cumulative impact of numerous small changes to many buildings.  Terraces that once displayed an integrity of design through the use of matching features such as doors, window, chimneys and porches, have been unbalanced by various alterations and additions.  On the whole such changes do not require planning permission.

In order to preserve and enhance the character of conservation areas, many planning authorities use Article 4 Directions to restrict permitted development rights on groups of buildings or areas.  Restrictions normally relate to particular elements such as replacement windows and doors, or roofing.

The character assessment does not identify a sufficient cluster of significant historic features to justify the introduction of an Article 4 Direction.  Currently, no Article 4 Directions are planned for this area.

Buildings at risk

It is intent of the Borough Council to take necessary action to secure repair & full use of any buildings at risk.  At the moment none of the listed buildings are at risk of decay and all appear to be in a good state of repair. The locally listed buildings are generally in good condition.

Review the boundary

The appraisal and comments made at the subsequent public meeting have identified a number of areas currently outside the conservation area, which make a significant contribution to its character.  It is intended to review of the boundary and a number of changes have been proposed to include:

Southfields Park wall & railings

Moira Street – north end

Include 86 - 94 Leicester Road

Inclusion of Great Central Road

Identify possible buildings for spot listing

None of the buildings within the conservation area were identified for “spot listing” by the character assessment.

Enhancement

There are a number of opportunities for enhancement:

- Visual improvements at Barrow Street and consideration of the removal of advertising hoardings will be carried out as part of the design and construction of the proposed Inner Relief Road.

- The possible extension to the conservation area would provide the opportunity to consider visual improvements at Moira Street and King Street junction, including the removal of advertising hoardings.

Proposals for developing an economic development & regeneration strategy for the area

Historic building repair grants are available from both Charnwood Borough Council and Leicestershire County Council.  Repair and reinstatement works to historic buildings, that make a vital contribution to maintaining and improving the character of the conservation area are likely to be eligible for grant assistance.

A commercial premises grant scheme is also run by the council to improve the external appearance of buildings.  An improvement in the appearance of an area through upgrading the buildings should lead to an overall increase in the number of people visiting, shops and restaurants, thus benefiting the commercial vitality of the area.

Strategy for the management & protection of important trees, greenery and green spaces

The Biodiversity Action Plan sets out the Borough Council’s priorities for conservation of a variety of wildlife and their habitats within Charnwood and details the actions required to bring about a reversal in its fortune.  The BAP will be reviewed and modified as required, to ensure that the necessary actions are being taken by the appropriate agencies.

Many of the significant trees along Leicester Road are mature or over mature.  It is important over the next few years to focus on replacement trees as this will help mitigate the eventual loss of some of these mature specimens.  The Council will provide guidance and advice on how this can be achieved and investigate sources of grant aid.

Proposals for an urban design and / or public realm framework

Visual intrusion from over engineered highway if Inner Relief Road is built

Exacerbated severance from the town centre

Loss of avenue of trees

Monitoring change arrangements

It is planned to review the conservation area character appraisal and its management plan every five year, although the management plan may under certain circumstances need to be reviewed over a shorter time period.  A photographic record of the conservation area has been made and will be used to help identify the need to review how changes within the conservation area are managed.  A greater degree of protection will be accomplished if the local community help monitor any changes.

Consideration of resources

This management plan sets out the commitment of the Borough Council to protecting the character and appearance of Charnwood’s conservation areas and how it will use its resources to achieve these aims.  Unfortunately, it is not possible to pursue all actions that may be seen as desirable but continued monitoring and review will help focus those resources to be used in the most effective way.

Timescale

Short Term (12 months)

- Produce a development brief for Hi Q Garage site

- Review how the current Shopfront guidance is used in order to promote improvements.

Medium Term (up to 3 years)

- Review Conservation Area Boundary

- Promote better use of the space at the junction of Gregory St/Moira St.

Long Term (over 3 years)

- Investigate visual improvements at Barrow Street including advertising hoardings

- Investigate visual improvements at King Street including advertising hoardings

Within 5 years the Council will undertake a review of this Appraisal and Management Plan in full consultation with local residents, businesses and land owners.  Circumstances may dictate that the review of the Management Plan is undertaken earlier.

Appendix 1 Local Plan Policies

POLICY EV/1 Design

- The Borough Council will seek to ensure a high standard of design in all new developments.  Planning permission will be granted for new development which:

- respects and enhances the local environment including the scale, location, character, form and function of existing settlements and the open and undeveloped nature of the countryside;

- is of a design, layout, scale and mass compatible with the locality and any neighbouring buildings and spaces;

- utilises materials appropriate to the locality;

- provides positive and attractive built frontages to existing or proposed public spaces including roads, footpaths, waterways and areas of public open space;

- safeguards important viewpoints, landmarks and skylines;

- uses the landform and existing features in and around the site, such as woodlands, trees, hedges, ponds, important buildings and structures imaginatively as the focus around which the new development is designed;

- safeguards the amenities of adjoining properties, particularly the privacy and light enjoyed by adjoining residential areas;

- meets the needs of all groups, including the disabled; and

- minimises the opportunity for crime to create a safe and secure environment.

POLICY EV/2 Nationally Important Archaeological Sites

- Planning permission will not be granted for development which would adversely affect a scheduled ancient monument or other nationally important archaeological site, or its setting.

POLICY EV/3 Archaeological Sites of County and Local Significance

- Planning permission for development affecting known archaeological sites of county or local significance, will be granted provided the archaeological interest can be preserved in situ.  Where this is not feasible or justifiable, the excavation and recording of the remains under the supervision of a professionally qualified archaeologist prior to development of that part of the site affected by the remains will be required.

POLICY EV/4 Alterations or Extensions to Listed Buildings

- Planning permission for alterations or extensions to listed buildings, where required, will only be approved where the work is in keeping with the special architectural or historic interest of the building or its character or setting.

POLICY EV/5 The Setting of Listed Buildings

- Planning permission for development which would adversely affect the setting of a listed building will be refused.  In granting planning permission for development near to a listed building the Borough Council will impose such conditions as it considers necessary to protect the building’s character and setting.

POLICY EV/6 Change of Use of Listed Buildings

- Applications for the change of use of listed buildings to ensure their continued viability will be approved provided they are in keeping with the character, appearance and historic interest of the building.  Applications must include full details of any proposed internal or external alterations to the building so that the effects on its character, appearance and setting can be fully assessed.

POLICY EV/7 Demolition of Listed Buildings

- Planning permission for development involving the demolition of the whole or substantially all of a listed building will only be granted where it can be demonstrated to the Council’s satisfaction that the condition of the building makes it impracticable to repair, renovate or adapt to a use which would ensure its retention.

POLICY EV/8 Buildings of Local Historic or Architectural Interest

- Planning permission for development which would affect a building of local historic or architectural interest or its setting will be granted provided:

- the appearance or character of the building and its setting are safeguarded; or

- the development would result in significant local community or environmental benefits.

POLICY EV/9 Historic Parks and Gardens

- Planning permission will not be granted for development which would have an adverse effect on the character or setting of the parks and gardens of historic or landscape significance as shown on the Proposals Map.

POLICY EV/10 Development in Conservation Areas

- In determining planning applications for development in designated conservation areas the Council will preserve or enhance their special character or appearance by:

- refusing permission for development involving the demolition of buildings unless it can be demonstrated that their condition is such that their repair, renovation or adaptation would be impracticable or that their removal or replacement would enhance the appearance or character of the area;

- requiring, where development would involve demolition, sufficient details of how the site will be treated, and of any scheme for redevelopment of the site, so that the effect of the proposal on the character of the conservation area can be properly assessed.  Conditions may be imposed to ensure redevelopment commences as soon as practicable following demolition;

- requiring the retention of materials, features and details of unlisted buildings or structures which contribute to the character of the conservation area;

- requiring the height, size, design, roofscape, plot width, visual appearance and materials used in any new development (including alterations and extensions to existing buildings) to respect the character of the surrounding area and make a positive contribution to the general character of the conservation area;

- requiring sufficient details of any proposal, including changes of use, so that the effect of the proposal on the character of the conservation area, both visually and in terms of any potential noise or other environmental intrusion, can be properly assessed.  Detailed proposals may also be required on sensitive sites adjoining a conservation area.  Alternatively design briefs will be prepared to guide development in the most sensitive locations;

- retaining the historic street pattern, traditional buildings lines, open spaces of special character (including gardens), other spaces of public value, important trees and water features, footways, footpaths and kerblines which contribute to the character of a conservation area; and

- encouraging the redesign or replacement of buildings, structures and features which are visually detrimental to the character of a conservation area.

POLICY EV/11 Advertisements

- Express consent for the display of advertisements and signs will be approved unless the proposal would be either:

- damaging to local amenities by reason of clutter, scale, siting, design or illumination; or

- dangerous to public safety through the introduction of signs likely to create a hazard by distracting or confusing road users.

POLICY EV/12 Advertisements Above First Floor Level

- Express consent will not be granted for the display of advertisements above the level of first floor window sills where they would be poorly related to the building or surrounding buildings by reason of their scale or design.

POLICY EV/13 Advertisements on Listed Buildings or in Conservation Areas

- Express consent to display advertisements and signs within conservation areas or on or close to a listed building will only be granted where:

- the design, scale and siting of the sign would preserve or enhance the character or appearance of the conservation area; and/or

- the proposal would not adversely effect the special architectural or historic interest of a listed building or its character or setting.

POLICY EV/18 Open Spaces of Special Character

- Planning permission will not be granted for development which would result in the loss of important areas of open land retained in public or private ownership which contribute to the character of a settlement either individually or as part of a wider network of open space.

POLICY EV/21 Sites of National Ecological or Geological Importance

- Planning permission will not be granted for development which could adversely affect National Nature Reserves or Sites of Special Scientific Interest (SSSI’s), proposed or designated for their ecological interest, unless an overriding national need can be shown which exceeds the level of importance for nature conservation and there is no alternative solution or other site suitable for that particular purpose.

- For Sites of Special Scientific Interest proposed or designated for their geological interest, planning permission will not be granted for development which could have an adverse affect unless an overriding national need can be shown which exceeds the level of importance for nature conservation or a suitable substitute site of at least equal value can be proposed.

POLICY EV/22 Sites of Regional, County and District Level Ecological or Geological Importance

- Planning permission will not be granted for development which could adversely affect County and District level sites of ecological interest or Local Nature Reserves unless an overriding strategic need can be shown which exceeds the level of importance for nature conservation.

- For Regionally Important Geological Sites and County and District level geological sites, planning permission will not be granted for development which could have an adverse affect unless an overriding strategic need can be shown which exceeds the level of importance for nature conservation or a suitable substitute site of at least equal value can be proposed.

POLICY EV/23 Sites of Parish Level Ecological or Geological Importance

- Planning permission for development in and around parish level ecological and/or geological sites will not be granted unless measures are included to protect or compensate for valuable habitats or features damaged or destroyed during the development.

POLICY EV/24 Landscape Features Important for Nature Conservation

- Planning permission for development which would affect a feature of the landscape of major importance for wild flora and fauna will be not be granted unless:

- the feature is safeguarded and retained within the development;

- its function as a linear and continuous corridor or stepping stone for migration, dispersal and genetic exchange is not diminished; and

- a scheme for the management of the nature conservation interest is prepared.

POLICY EV/25 Development and Features of Nature Conservation Interest

- Proposals for new development will be expected to include measures to conserve existing features of nature conservation interest and to include proposals for the creation of new habitats where appropriate.

POLICY EV/26 Species Protection

- The Borough Council will not grant planning permission for a development that would have an adverse effect upon any site supporting badgers, bats and all other species protected by schedules 1, 5 and 8 of the Wildlife and Countryside Act 1981, as amended, or Annex IV to the EC Habitats Directive unless measures are included in the proposal to protect the species and minimise any disturbance during development, or to provide for its transfer to an alternative site of equal value.

POLICY EV/38 Satellite Television Dishes

- Proposals for satellite dishes which require planning permission will be acceptable provided the dish:

- by reason of its size, colour, appearance and siting would not have a significant impact on the appearance of the building;

- would not be visually intrusive or damaging to the wider streetscene;

- would not adversely affect the amenities of neighbouring properties;

- would not lead to a clutter of dishes which collectively would detract from the character and appearance of the locality.

POLICY CA/11 Use of Upper Floors

- Within Loughborough Town Centre and its associated service areas, district centres and local centres planning permission will be granted for the following uses (as defined by the Town and Country Planning (Use Classes) Order 1987) of upper floors, subject to the retention of residential amenities where adjacent property continues to be occupied:

- shops (Class A1);

- financial and professional services (Class A2);

- food and drink uses (Class A3);

- offices (Class B1(a));

- dwellings (Class C3);

- non residential institutions (Class D1); and

- assembly and leisure uses (Class D2).

- Planning permission for new built development within the centres listed above will be granted provided that the development is of a scale appropriate to the centre it seeks to serve, and subject to the provisions set out above and to the inclusion of positive proposals for the use of premises at first floor and above which will contribute to the vitality and viability of the centre.  In particular elements of housing should be provided wherever possible.

POLICY CA/12 Shop Front Design

- In granting planning permission for shop fronts in new buildings the Borough Council will require that they be designed as an integral part of the overall frontage having regard in particular to the style, materials and proportions of the building and to the overall character of the area.

- Replacement shop fronts will be permitted provided they maintain or improve the appearance of the frontage into which they will be placed and relate well to surrounding buildings and the wider street scene.  Shop fronts will be required to reflect the architectural composition of the existing frontage, particularly where a commercial ground floor user crosses several facades of differing character.

POLICY CA/13 Shop Front Security

- Planning permission for the fitting of roller shutters and security grills will be granted provided that all the fittings are colour coated to match the shop front, designed to be unobtrusive during hours of business and allow visibility into the shop when in place.